Contents

1.	Scope	2

2.	Connector Definitions	2

2.1	Connector TBD	2

2.2	Digital ICD	2

3.	Communication format	4

3.1	SPI Interface Specifications	4

3.1.1	SPI write transaction	4

3.1.2	SPI Read transaction	5

3.1.3	Timing constrains	5

4.	SPI Command set	6

4.1	MOFC operation mode (Determined by OPMODE Configuration Register)	6

4.2	Configuration flow (3 steps)	7

4.3	Indirect read/write to tuner filter SPI	8

5.	Command details	9

5.1	status	9

5.2	FPGA_VER_REG	10

5.3	SERIAL_NUMBER	10

5.4	TEST_REG	10

5.5	Global config Reg	11

5.6	Bit config Reg	11

6.	Other Design requirements/specificatiosn	12

6.1	SPI back to back transaction	12

6.2	Maximum Slave processing time from receiving LOAD command to RF RDY	12

Figures

No table of figures entries found.

Tables

No table of figures entries found.

# Scope

This document defines the SPI interface and protocol for configuring and controlling the MOFC (Miniature Octal Frequency Converter) device. The purpose of this document is to:

- Establish a clear communication protocol between the MASTER and SLAVE devices over SPI.

- Define the command structure for reading from and writing to configuration registers.

- Provide guidelines for the implementation of SPI-based control in the FPGA firmware.

- Detail the electrical and timing requirements for SPI communication.

- Outline the process of configuring and updating the RF module of the MOFC device.

This document serves as a reference for both the firmware development team implementing the SPI interface and the hardware design team ensuring compliance with the interface specifications

# Connector Definitions

## Connector TBD

## Digital ICD

| Name | Function | Type |
| --- | --- | --- |
| SPI_Clk_N | SPI Clock | RS485 input |
| SPI_Clk_P |  |  |
| SPI_CSn_N | Chip Select active low Chip Select active low | RS485 input RS485 input |
| SPI_CSn_P |  |  |
| SPI_MOSI_N | Master Out, Slave In | RS485 input |
| SPI_MOSI_P |  |  |
| SPI_MISO_N | Master In, Slave Out | RS485 output |
| SPI_MISO_P |  |  |
| Resetn_N | System Reset active low | LVTTL 3.3V Input |
| Resetn_P |  |  |
| RF_RDY_N | Indicates the tuner RF is ready | RS485 output |
| RF_RDY_P |  |  |

Table 1: Digital ICD

# Communication format

The MOFC controller acts as the “SLAVE” and the main controller acts as the “MASTER”. The MOFC controller uses a typical SPI interface in terms of logic, but uses differential RS485 inputs/outputs. The SPI interface consists of four lines: SClk, CSn, MOSI, and MISO. The SPI interface uses 32-bit commands. All write commands transmitted to MOFC must be 32-bits long. Read commands contain the R/W bit set to 1 and the register address on MOSI while the returning data is clocked out on the MISO line during the data portion of the same 32-bit sequence.

## SPI Interface Specifications

The specified clock frequency for this application is 10MHz. Only the MASTER controller supplies the SPI 10MHz clock. The 10MHz clock is only supplied during SPI transactions.

### SPI write transaction

A typical Write transaction is shown below. The new command goes out on the MOSI line, and the data is registered by the SLAVE on each positive clock edge. The R/W bit is set to 0. The transaction is initiated when the MASTER pulls the CSn line low for a period of Tcs_c before the first positive clock edge and held low for the remainder of the transaction. The MASTER then pulls the CSn line high Tc_cs after the last negative clock edge. See timing constraints in the table below for minimum timing specifications.

Figure 1: SPI write transaction

### SPI Read transaction

A typical Read transaction is shown below. The Read transaction sequence starts out with the MASTER clocking out the R/W bit set to 1 and clocking out the address bits on each positive clock edge. The SLAVE then starts clocking out the read data on the next clock falling edge. The MASTER registers the 24 bits of data on the remaining 24 positive clock edges.

Figure 2: SPI read transaction

### Timing constrains

| Parameter | Description | Min | Max |
| --- | --- | --- | --- |
| SPI Clock Frequency | Maximum frequency of SPI Clock | 10 MHz |  |
| Tcs_c | Time between CSn going low and the first positive clock edge | TBDns |  |
| Tc_cs | Time between last negative clock edge and CSn going high | TBDns |  |

Table 2: Timing consrains

# SPI Command set

Table 3.0 contains a list of SPI commands/registers used to communicate with the MQFC. For details about each command, refer to the section 5, “Command Details”.

Table 3: SPI commands

## MOFC operation mode (Determined by OPMODE Configuration Register)

- The OPMODE register dictates how channel configurations are applied. This register is project-specific

  - Single Channel Mode (0x0): Each of the eight channels is individually configured by the master.

  - Common Quad Mode (0x1): The master fills configurations for channels 1 and 2. Channel 1’s configuration is used for channels 2, 3, and 4, and channel 2’s configuration is used for channels 6, 7, and 8.

  - Common Octal Mode (0x2): The master configures only channel 1. This configuration is then applied to all remaining channels (2-8).

## Configuration flow (3 steps)

The configuration process proceeds in three optional stages.

- Stage 1: Configuration Data Loading

  - Master Action: The master writes data to the following:

    - Bit Configuration Register

    - Channel Configuration Registers (data written depends on the selected OPMODE)

      - When OPMODE = 0x0 : channels 1-8 are configures by master

      - When OPMODE = 0x1: channels 1,2  are configured by master

      - When OPMODE = 0x2 : only channel 1 is configured by master

  - Slave Action: No response is required from the slave during this stage. This stage prepares all necessary configuration information within the registers.

- Stage 2:  Checksum Verification

  - Master Action: The master writes to the Checksum Register.

  - Slave Action: The slave initiates a checksum verification process. This process uses the Bit Configuration Register and Channel Configuration data (applied according to the selected OPMODE). The slave then updates the Checksum Status (CK_STAT field at Status register) to either “Pass” or “Fail.”

- Stage 3:  RF Setup Loading

  - Master Action: The master writes to the Load RF Setup Register.

  - Slave Action: The slave performs the following:

    - Reset the CK_STAT field to "IDLE"

    - De assert RF_RDY to '0'

    - assigns the MOFC based on the loaded configuration.

    - Wait until all assignment are completed and all PLL are locked and then assert RF_RDY to '1'

## Indirect read/write to tuner filter SPI

The INDIRECT_TF_ACCESS register enables read or write access to the tunable filter via SPI. Writing to this register initiates a transaction with the target tunable filter based on the specified parameters.

Register Fields:

- CHID and TFID: Specify the target tunable filter.

- Access Type: Determines the operation type — Write (0) or Read (1).

- TF SPI Address: Specifies the SPI register address in the tunable filter to be accessed.

- Write Data: Data to be written. (Ignored during a read operation.)

Operation:

Writing to INDIRECT_TF_ACCESS triggers the slave to access the selected tunable filter and perform the specified SPI operation.

- For Write Access (Access Type = 0):

  - Sends the Write Data to the specified SPI address in the selected tunable filter.

  - The 8-bit result from the SPI MISO line is copied to the TF_READ_VALUE register.

- For Read Access (Access Type = 1):

  - Reads from the specified SPI address of the selected tunable filter.

  - The 8-bit result from the SPI MISO line is copied to the TF_READ_VALUE register.

# Command details

## status

| Param | Bits | Description | Possible Values |
| --- | --- | --- | --- |
| R/W | 31 | Read / Write bit | Read only |
| Address | 30:24 | Register address bits | 0x0 |
| Temperature | 23:16 | Module temperature is 2’s complement format | C9 = -55°C …7D = 125°C |
| PS | 15 | Power status | 0-Fail 1-Pass |
| CK_STAT | 13:14 | CheckSum Status, this bit will be updated when receiving command CHECKSUM, | 0x0-CheckSum OK 0x1- CheckSum error 0x2- CheckSum success |
| RFRDY | 12 | RF ready, is asserted low after receiving command ‘LOAD_RF_SETUP’, and only after complete setup of tuner is asserted High, the RFRDY out is also reflected at the output at PIN READY |  |
| RSVD | 11 | Unused bits | 0x0 |
| LO2_LCK | 10 | LO2 pll lock | 0 – unlocked: 1-Locked |
| ALCK | 9 | Clock toward ADC lock | 0 – unlocked: 1-Locked |
| BLCK | 8 | Bit PLL lock | 0 – unlocked: 1-Locked |
| LCK8 | 7 | Subunit 2 LO4 lock indication | 0 – unlocked: 1-Locked |
| LCK7 | 6 | Subunit 2 LO3 lock indication | 0 – unlocked: 1-Locked |
| LCK6 | 5 | Subunit 2 LO2 lock indication | 0 – unlocked: 1-Locked |
| LCK5 | 4 | Subunit 2 LO1 lock indication | 0 – unlocked: 1-Locked |
| LCK4 | 3 | Subunit 1 LO4 lock indication | 0 – unlocked: 1-Locked |
| LCK3 | 2 | Subunit 1 LO3 lock indication | 0 – unlocked: 1-Locked |
| LCK2 | 1 | Subunit 1 LO2 lock indication | 0 – unlocked: 1-Locked |
| LCK1 | 0 | Subunit 1 LO1 lock indication | 0 – unlocked: 1-Locked |

## FPGA_VER_REG

| Param | Bits | Description | Possible Values |
| --- | --- | --- | --- |
| R/W | 31 | Read / Write bit | Read only |
| Address | 30:24 | Register address bits | 0x1 |
| VER_ID | 23:16 | Version ID represented by 2 hexa digits | 0x01 – 0xff |
| VER_DAY | 14:10 | FPGA firmware date - day | 1 to 31 |
| VER_MONTH | 9:6 | FPGA firmware date - year | 1-12 |
| VER_YEAR | 5:0 | FPGA firmware date - year | 24-63 |

## SERIAL_NUMBER

| Param | Bits | Description | Possible Values |
| --- | --- | --- | --- |
| R/W | 31 | Read / Write bit | Read only |
| Address | 30:24 | Register address bits | 0x2 |
| rsvd | 23:20 | Unused bits | 0X0 |
| SER_NUM | 19:0 | Module serail number | 0x00 – 0xfffff |

## TEST_REG

| Param | Bits | Description | Possible Values |
| --- | --- | --- | --- |
| R/W | 31 | Read / Write bit | Read only |
| Address | 30:24 | Register address bits | 0x3 |
| DATA | 23:0 | Register purpose is just for testing the complete 24 bits can be write/write by master in order to test basic communication | 0x00 – 0xffffff |

## Global config Reg

| Param | Bits | Description | Possible Values |
| --- | --- | --- | --- |
| R/W | 31 | Read / Write bit | Read only |
| Address | 30:24 | Register address bits | 0x4 |
| RSVD | 19:0 | Unused bits | 23:2 |
| OPMODE | [1:0] | Operation mode | 0x0: 2G_PHS_MODE 0x1:4G_PHS_MODE 0x2:16G AMP MODE |

## Bit config Reg

| Param | Bits | Description | Possible Values |
| --- | --- | --- | --- |
| R/W | 31 | Read / Write bit | Read only |
| Address | 30:24 | Register address bits | 0x5 |
| RSVD | 22 | Unused bit | 0x0 |
| P/CW | 22 | Select between Pulse or CW | 0 - BIT 1 - CW |
| BIT_DCA | 21:16 | BIT Attenuation, each lsb is 0.5[db] range is 0 to 31.5[db] | 0x0-0x3F |
| BIT_FREQ | 15:0 | Select BIT frequency, LSB unit [TBD] range [TBD] | TBD |

# Other Design requirements/specificatiosn

## SPI back to back transaction

Back to back transaction : 10 SPI clock CYCLES between access

## Maximum Slave processing time from receiving LOAD command to RF RDY

Design goal TBD[us]