    - MOFC control as agreed with GAY, BORIS ,  AT JULY 04

    - Controls –J14

| Port | From | Port Function | Connector Type |
| --- | --- | --- | --- |
| J8 | CSRP | System Controls | D38999/20WC35PN (22 PIN) |

Table 6 External Control Definition from CSPIR

| Function | Signal Name | PIN | Signal Type |
| --- | --- | --- | --- |
| RS422 (485) NADCOM | SPI_Clk_N | 1 | RS422_IN |
|  | SPI_Clk_P | 2 |  |
|  | SPI_CSn_N | 3 | RS422_IN |
|  | SPI_CSn_P | 4 |  |
|  | SPI_MOSI_N | 5 | RS422_IN |
|  | SPI_MOSI_P | 6 |  |
|  | SPI_MISO_N | 7 | RS422_OUT |
|  | SPI_MISO_P | 8 |  |
|  | Resetn_N | 9 | RS422_IN |
|  | Resetn_P | 10 |  |
|  | RF_RDY_N | 11 | RS422_OUT |
|  | RF_RDY_P | 12 | RS422_OUT |
| Rs232 Debug | RS232 PCC TX_H | 13 | RS232 OUT |
|  | RS232 PCC TX_L | 14 | RS323 IN |
|  | GND | 15 | GND |
|  | DEBUG_ENABLE | 16 | TTL IN |
| SPARE | SPARE1-H | 17 | RS422_OUT |
|  | SPARE1-L | 18 |  |
|  | SPARE2-H | 19 | RS422_IN |
|  | SPARE2-L | 20 |  |
|  | SPARE3-H | 21 | RS422_IO |
|  | SPARE3-L | 22 |  |

טבלה מהמסמך של גיא

| Name | Function | Type |
| --- | --- | --- |
| SPI_Clk_N | SPI Clock | RS485 input |
| SPI_Clk_P |  |  |
| SPI_CSn_N | Chip Select active low Chip Select active low | RS485 input RS485 input |
| SPI_CSn_P |  |  |
| SPI_MOSI_N | Master Out, Slave In | RS485 input |
| SPI_MOSI_P |  |  |
| SPI_MISO_N | Master In, Slave Out | RS485 output |
| SPI_MISO_P |  |  |
| Resetn_N | System Reset active low | LVTTL 3.3V Input |
| Resetn_P |  |  |
| RF_RDY_N | Indicates the tuner RF is ready | RS485 output |
| RF_RDY_P |  |  |