Specification

For

MOFC (0.5-18)

" Miniature Octagon Frequency Converters”

| Record of Changes and Revisions Original document (if created from another document): |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| VER. | REV. | ECR No. | DESCRIPTION | DATE | APPR. |
| 001 | - | - | Release | 11/23 | N. Moshe |
| 002 |  |  | Updated for electrical section | 29/OCT 2024 | Mr. JEON |

| First Revision Approvers |  |  | Author: | B. Yamin | Checker: | N. Moshe |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Conf. Mgr.: | A. Tamis |  | Q.A.: | ZMIRI | Final Approver: |  | N. Moshe |
| Other (Specify Role): |  |  |  |  |  |  |  |

| Current Revision Approvers |  |  | Author: | B. Yamin | Checker: | N. Moshe |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Conf. Mgr.: | A. Tamis |  | Q.A.: | ZMIRI | Final Approver: |  | N. Moshe |
| Other (Specify Role): |  |  |  |  |  |  |  |

Table of Contents

1.	Scope	8

1.1	UNITS - P/N	8

1.2	Standards	8

1.3	Abbreviations:	9

1.4	Definitions	10

1.4.1	Tracking	10

1.4.2	Matching	10

1.4.3	Switch Isolation	11

1.4.4	RF Isolation	11

1.4.5	Switching Speed	11

2.	Applicable document	12

2.1	Specifications	12

2.2	Standards	13

2.3	Document No.	14

2.3.1	Elta Documents	14

2.3.2	Vendor Documents	14

3.	REQUIREMENTS	15

3.1	General	15

3.1.1	Important design considerations:	15

3.1.2	system setting capability	15

3.2	MOFC    RF – input output:	16

3.3	Operational system mode	16

3.4	Direct or conversion frq. Setting	16

3.5	Principle block diagram	17

3.5.1	With internal FE – one channel	17

3.5.2	Without input FE - one channel	17

3.5.3	Principle frequency conversion	17

3.5.4	ALL channels - With input FE	18

3.6	"RF MOFC   " functionality:	19

3.7	mode of operation – 3 modes	20

3.7.1	Amp mode @16GHz BW	20

3.7.2	PHASE MODE BW@ 2GHz	21

3.7.3	PHASE MODE BW@ 4GHz	22

3.8	RF Ports IN and OUT	24

3.9	MOFC    RF Voltage & Control – TBD	25

3.9.1	Control and SPI connector	25

3.9.2	JTAG – hidden MDM connector	26

3.10	Electrical Requirements	27

3.10.1	one channel Requirements	28

3.10.2	8 RF path Electrical unit Requirements  phase match or stand alone at separated frq.	32

3.10.3	Local Oscillators / Reference	34

3.10.4	CAL RF Source output port	35

3.10.5	Clock to A/D	36

3.10.6	BIT (Built in Test)	36

3.11	Mechanical requirements	37

3.11.1	Outline Drawing	37

3.11.2	RF Connectors:	37

3.11.3	DC and control connector:	37

3.12	Weight	38

3.13	Design and contraction	38

3.13.1	Material and Parts	38

3.13.2	SOLDER LEAD FREE	38

3.13.3	Aluminum Alloys	38

3.13.4	Brass	38

3.13.5	Dissimilar Metals	39

3.13.6	Fungus Inert Material	39

3.13.7	Arc-Resistant Materials	39

3.13.8	Flammable Materials	39

3.13.9	Adhesives	39

3.13.10	Stripline Transmission Lines	39

3.13.11	ESD (Electro-Static Discharge)	39

3.13.12	Threaded Parts:	39

3.13.13	Soldering	39

3.13.14	Finish	39

3.13.15	Marking	40

3.13.16	Name plate marking	40

3.13.17	Resistance to solvents:	40

3.13.18	Resistance to soldering heat	40

3.13.19	Operating Positioning:	40

3.14	Environmental Requirements	41

3.14.1	General Environmental specification	42

3.14.2	Environmental testing and analysis	45

3.14.3	Environmental Requirements	45

3.15	EMI and EMC	48

3.15.1	Bonding	48

3.15.2	Bonding Unit's body:	48

3.15.3	Bond for shielding:	48

3.15.4	Bond for connectors	48

3.15.5	Bond for EMI filters	48

3.15.6	Electromagnetic Compatibility	49

3.16	Reliability and Quality Requirements	50

3.16.1	Predicted MTBF	50

3.16.2	Storage Life	50

3.16.3	Operating Service Life	50

3.16.4	Maintainability	50

4.	QUALITY ASSURANCE PROVISIONS	51

4.1	Responsibility for inspection	51

4.1.1	Source Inspection	51

4.1.2	Non-Performance of Test/Inspections	51

4.1.3	Test Equipment and Inspection Facilities	51

4.2	Acceptances Test	51

4.2.1	ESS	52

4.2.2	Thermal Cycles	52

4.2.3	Random Vibration	53

4.2.4	Mechanical and Visual Inspection	54

EMC and EMI Assurance and Tests	55

4.2.5	Bonding	55

4.2.6	Electromagnetic Compatibility	55

5.	RESPONSIBILITY OF SELLER	56

5.1	Implementation of changes	56

6.	PREPARATION FOR DELIVERY	56

7.	APPROVED SOURCES	56

8.	VERIFICATION MATRIX	57

Table of Figure

Figure 1: Difference between variations	9

Figure 2: Difference between variations	9

Figure 3: one channel with internal FE	16

Figure 4: one channel – without internal FE	16

Figure 5: Block Diagram–Eight channels and Clock module	17

Table of Table

Table 1:  MOFC	7

Table 2: RF IN from FE /Antenna & Ref 10MHz	23

Table 3: RF OUT ports to CSPIR (A/D) - OUTPUT	23

- Scope

This specification establishes and covers the general requirements which must be met in the procurement of the MOFC    "miniature Octagon Frequency Converter” LRU.

  - UNITS - P/N

Table 1:  MOFC

| No | Main Function | Name | Elta P/N |
| --- | --- | --- | --- |
| 1 | Miniature Octagon Frequency Converters | MOFC | 3019Y200-001 |

Classification

The MOFC   is intended for mobile Ground/airborne systems and shall meet the requirements specified herein.

  - Standards

The following documents, of the issue in effect on the date of the issue of this specification, unless otherwise specified, form a part of this specification to the extent specified herein. In the event of any conflict between the documents referenced herein and the contents of this specification, the latter shall govern.

  - Abbreviations:

ATE 	- 	Automatic Test Equipment.

ATP 	- 	Acceptance Test Procedure.

CAL 	- 	Calibration.

CW 	- 	Continuous wave.

DVT 	-	Design Verification Test

ESS 	-	Environment Stress Screening.

FOV  	- 	Field Of View

Gold 	-	Reference matrix

ICD 	- 	Interface Control Document.

IL 	-	Insertion Loss

LRU 	-	Line Replacement Unit

M.P.	-	Multi Port connector

NGT 	-	Not Greater Than

PET 	-	Phase Tracking

POP	-	Phase Output Port

Ptp 	- 	Peak To Peak

PW 	- 	Pulse Width.

PWR 	- 	power.

QA 	- 	Quality Assurance

QTP 	 	Qualification Test Procedure.

SOCD	- 	Source Control Drawing or Specification Control Document

SOR 	-	Set On Ram

STD 	- 	Standard.

S/W 	- 	software

TBD 	- 	To Be Defined.

VCD 	-	Vendor Control Document/Drawing

W.C. 	-	Worst Case (w.c.)

  - Definitions

    - Tracking

Phase or Amplitude Tracking over  temperature.

The peak to peak value of the difference between two measurements performed at different temperatures.

Figure 1: Difference between variations

Diff = dP1-dP2

    - Matching

Phase or Amplitude matching over frequency

The peak to peak value of the difference between two measurements performed at different channels

Figure 2: Difference between variations

Diff = dP1-dP2

    - Switch Isolation

It is the difference in the output power between the two following cases:

- The switch is selected to "on" state.

- The switch is selected to other states.  (i.e. "off" or other ports are selected, if applicable)

    - RF Isolation

It is the Insertion Loss between one port (in which a signal of frequency F1 is required) to another port (in which that signal of frequency F1 is not a desired signal).

In example, it is the Insertion Loss in a path that the switch in that path is in OFF position.

    - Switching Speed

The time from application of control signal (i.e., the time it reaches 50% of its final logic level), to the time of RF output

The following values are achieved: 
For "OFF" to "ON": the gain value reaches its final value 0.25 dB, AND (if phase is required) the phase reaches its final requirements ±1º
For "ON" to "OFF":   5 dB from the final value.

Notes:

The control input signal, controls by one of the two options:

- Direct control:  the input signal controls the device directly.

- Strobe control:  the input signal control a logic circuit when data and control are present.

Note: the switching speed is relevant to switch devices, DCA’s and any other devices controlled by logic signals.

- Applicable document

  - Specifications

Federal

QQ-A-200	Aluminum Alloy, Bar, Rod, Shapes, Structural Shapes, Tube and Wire, Extruded: General Specification For.

QQ-A-250 	Aluminum and Aluminum Alloy Plate and sheet: General Specification For.

QQ-A-596	Aluminum Alloy Permanent and Semi-Permanent Mold Castings

QQ-A-601	Aluminum-Alloy Sand Castings

QQ-A-626	Brass, Leaded and Non-leaded

Military

MIL-STD-1275D

MIL-S-3928 	Switches, RF transmission Line, General Specification for

MIL-B-5087	Bonding, Electrical and Lighting Protection for aerospace systems

ISO-9001		Quality System

MIL-F-14072 	Finishes for Ground Electronic Equipment

MIL-C-39012	Connector, Coaxial, Radio frequency, General Specification for

MIL-C-45662	Calibration System Requirements

MIL-C-11866	Casting, Precision, Nonferrous

MIL-P-13949	Plastic Sheet, Laminated, Metal Clad

  - Standards

Federal

FED-STD-H28	Screw Thread Standard for Federal Services

Military

MIL-STD-1275D	Characteristics of 28 Volt DC Input Power to Utilization Equipment in Military Vehicles

MIL-HDBK-454	Standard General Guidelines for Electronics Equipment

MIL-STD-461E	Requirements for the Control of Electromagnetic Interference Characteristics of Sub Systems and Equipment

MIL-STD-810 F/G	Environmental Test Methods

MIL-STD-883	Test Methods and Procedures for microelectronics

MIL-STD-889	Dissimilar Metals

MIL-STD-1276	Leads for Electronic Components

MIL-STD-1285	Marking for Electrical and electronics parts

MIL-HDBK-217F	Reliability Prediction of Electronic Equipment

MIL-E-4158E	Electronic Equipment, ground, general specification

International

| Document Ref. No. | Procedure ID |
| --- | --- |
| IPC-A-600F | Acceptability of Printed Boards |
| IPC-A-610C | Acceptability of Electronic Assemblies |
| J-STD-001C | Requirements for Soldered Electrical and Electronic |

Non Government

RTCA-DO/160D/F   	Environmental Conditions and Test Procedures for 	Airborne Equipment (Section 22, Waveform 3, Level 3)

  - Document No.

    - Elta Documents

| Name | UNIT. P/N | SPEC P/N | SOW P/N | Elta Dwg. [ICD] | QTP P/N |
| --- | --- | --- | --- | --- | --- |
| MOFC | 3019Y200-001 | 3019Y201-001 | 3019Y202-001 |  | 3019Y207-001 |

CSPIR: 3019Y330-001

    - Vendor Documents

| Name | Vendor Dwg. [SOCD] |  |  |  | QTR P/N |
| --- | --- | --- | --- | --- | --- |
| MOFC | 3019Y200-001 |  |  |  |  |

- REQUIREMENTS

  - General

The "MOFC   " units shall meet or exceed the electrical performance requirements under any combination of the herein specification requirements (Environmental, EMC, Setting, Etc).

The unit shall meet the following characteristics under:

- Any constant Temperature

- Any constant power level

- Any combination of different input power level in of 3dB window.

    - Important design considerations:

- Small and modular

- Phase & Amplitude matching between paths

  - Internal phase/amp lookup table

- Undesired signal –

  - The IF Filter 2.75 – 4.75 GHz should be high rejection performances

    - system setting capability

- Setting each channel independently

  - Frequency

  - Gain

MOFC    RF – input output:

Inputs:

- 8 input Antennas or Matrices  (phase match)

- Ref signal 10MHz (coax).

- Voltages (low dc voltage)

- Jtag (hidden)

Outputs:

- BIT out

- To the CSPIR (A/D)

    - SPI

    - 8 IF outputs phase match

    - REF 250 MHz (coax )

- Indications (LVTTL) – 4 Spare I/O (through CTL Connector)

Operational system mode

- Amp mode @16GHz BW

- Phase Mode BW@ 2GHz

- Phase Mode BW@ 4GHz

Direct or conversion frq. Setting

- 0.5-2 GHz -  Direct

- 2-18 GHz - frq. Conversion

  - Principle block diagram

The MOFC design to Receive signals from the 8 antennas or FE, and feed the processor Unit

    - With internal FE – one channel

Figure 3: one channel with internal FE

    - Without input FE - one channel

Figure 4: one channel – without internal FE

    - Principle frequency conversion

    - ALL channels - With input FE

Figure 5: Block Diagram–Eight channels and Clock module

"RF MOFC   " functionality:

Main function - The MOFC design to Receive signals from the 8 antennas or FE, and feed the processor Unit

- The MOFC   include the following functions 8 RF channel down converter- phase match

  - All 8 channels will set to the same frequency simultaneously

  - Each 4 channels can be set to a different frequency independently

  - Each channel can set to a different frequency independently

- The unit will be design to allowed a flexibility assembly configuration

  - Full - with internal FE

  - Without input FE – the external FE will be amplified by 15dB

  - 8 or 4 channels

- Clock module – 250 MHz sinus

- Synthesizer for external Cal/Bit use include MOD power & power control.

- Logic card

- Communication - SPI Serial

- Statues report: temp, S/W Ver, 11 LO lock.

mode of operation – 3 modes

Amp mode @16GHz BW

- Each channel set to a different rf input frequency

- different Fix LO for each channel

Figure 6: Amp mode @16GHz BW

PHASE MODE BW@ 2GHz

- 8 cannels setting to the same FRQ @ 2 GHz BW

- Same LO FRQ for the the 8 channels (two sub unit)

- 8 channels phase match

Figure 7: PHASE MODE BW@ 2GHz

PHASE MODE BW@ 4GHz

- 1-4 channels of sub unit #1 setting to the same FRQ @ 2 GHz BW

- 5-8 channels of sub unit #2 setting to the same FRQ @ 2 GHz BW

- sub unit #1 and sub unit #2 setting independently

- each sub unit (4 channels) phase match

Figure 8: PHASE MODE BW@ 4GHz

Figure 9: Preliminary Mechanical concept – "MOFC"

To be updated

RF Ports IN and OUT

Table 2: RF IN from FE /Antenna & Ref 10MHz

| Port | Connect to | Port Function | Connector Type Multiport |  |
| --- | --- | --- | --- | --- |
| J1-1 | MTX CH#1 | MOFC CH#1 | BMA MALE | IN |
| J1-2 | MTX CH#2 | MOFC CH#2 | BMA MALE | IN |
| J1-3 | MTX CH#3 | MOFC CH#3 | BMA MALE | IN |
| J1-4 | MTX CH#4 | MOFC CH#4 | BMA MALE | IN |
| J1-5 | MTX CH#5 | MOFC CH#5 | BMA MALE | IN |
| J1-6 | MTX CH#6 | MOFC CH#6 | BMA MALE | IN |
| J1-7 | MTX CH#7 | MOFC CH#7 | BMA MALE | IN |
| J1-8 | MTX CH#8 | MOFC CH#8 | BMA MALE | IN |
| J9 | 10MHz | MOFC CH#8 | SMA FEMALE | IN |
| J10 | Matrix | . (BIT/CAL) provision for | SMA FEMALE | OUT |

To be updated

Table 3: RF OUT ports to CSPIR (A/D) - OUTPUT

| Port | Connect to | Port Function | Connector Type | Cable Type |
| --- | --- | --- | --- | --- |
| J5 | A2D #1 | IF CH #1 | GPO FEMALE | OUT |
| J6 | A2D #2 | IF CH #2 | GPO FEMALE | OUT |
| J7 | A2D #3 | IF CH #3 | GPO FEMALE | OUT |
| J8 | A2D #4 | IF CH #4 | GPO FEMALE | OUT |
| J9 | A2D #5 | IF CH #5 | GPO FEMALE | OUT |
| J10 | A2D #6 | IF CH #6 | GPO FEMALE | OUT |
| J11 | A2D #7 | IF CH #7 | GPO FEMALE | OUT |
| J12 | A2D #8 | IF CH #8 | GPO FEMALE | OUT |
| J13 | A2D 250 MHz | 250 MHz | GPO FEMALE | OUT |

To be updated

MOFC    RF Voltage & Control – TBD

Control and SPI connector

Table 4: J9 – Will Be Defined

Connector Type: TBD

| Port | Connect to | Port Function | Signal Type |  |  | Port | Connect to | Port Function | Signal Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PS | 3.3V_RF (3A) |  |  |  | 14 | PS | -5V_RF (0.05A) |  |  |
| 2 | PS | 5V_RF (4A) |  |  |  | 15 | PS | 3.3V_RF (3A) |  |  |
| 3 | PS | 5V_RF (4A) |  |  |  | 16 | PS | 5V_RF (4A) |  |  |
| 4 | PS | GND_RF |  |  |  | 17 | PS | GND_RF |  |  |
| 5 | PS | 12V_RF(0.05A) |  | 0.05A |  | 18 | PS | GND_RF |  |  |
| 6 |  |  |  |  |  | 19 | PS | 3.3V_Essential (0.3A) |  | REF 250MHz |
| 7 | PS | GND_RF |  |  |  | 20 | PS | GND_RF |  |  |
| 8 | Digital | RESET_in | LVTTL | 3.3V |  | 21 | Digital | GNDD |  |  |
| 9 | Digital | SPI_MOSI_in | LVTTL | 3.3V |  | 22 | Digital | SPI_MISO_out | LVTTL | 3.3V |
| 10 | Digital | SPI_CLK_in | LVTTL | 3.3V |  | 23 | Digital | SPI_CSN_in | LVTTL | 3.3V |
| 11 |  | RF Rdy_out | LVTTL | 0=ok |  | 24 |  | Load (busy) Rdy out | LVTTL | 0=BUSY |
| 12 | Digital | PLL Lock _out (LO,BIT,250MHz | LVTTL | 0=ok |  | 25 | Digital | COM out | LVTTL | 0= Ready |

Note #1: No.1~7 and No14~20 will be changed as +28V with 3 pins

Note #2: DC con will be located in front side around RF input area

Note #3: Control pin with 15pin must be defined in detail

JTAG – hidden MDM connector

The Main controller & Power Supply JTAG connector is a separate connector only for change FPGA Ver.

This connector will be hiding by a removable cover.

The connector access should be directly without any LRU disassembly

Table 5:  JTAG Definition

| Function | Pin No | Signal Name | Signal Type |
| --- | --- | --- | --- |
| JTAG | 1 | JTAG_TDO | TTL |
|  | 2 | JTAG_TMS | TTL |
|  | 3 | JTAG_TCK | TTL |
|  | 4 | JTAG_TDI | TTL |
|  | 5 | +3.3V_CON | Voltage |
|  | 6 | NC | NC |
|  | 7 | NC | NC |
|  | 8 | NC | NC |
|  | 9 | +3.3V_RTN | GND |

  - Electrical Requirements

The performance should comply at any FRQ and temperature

The performance should comply gain setting =20dB

| Parameter | Requirements | Remark |
| --- | --- | --- |
| - one channel Requirements |  |  |
| - Input Frequency Range [GHz] | 0.35-18GHz |  |
| - Conversion IF Output Frequency Range - | IF Range: 2.75 to 4.75 GHz IF CF@ BW: 3.75±1 GHz Or IF Range: 0.4 to 2.40 GHz (System operating point of view) ( Only if required ) | Problematics singular IF frequency 2.4 to 2.6 GHz 4.9 to 5.1 GHz *) The IF FRQ is a result from the A/D Nyquist zone |
| - Conversion IF BW | 2 GHz | Gain reduction at IF Band Edge: 5dB |
| - Direct IF output Frequency Range - | 0.35 to 2.45 GHz (Filter requirement) or 2.75 to 4.75GHz |  |
| - Direct IF BW (sub harmonic BW) | >300 MHz |  |
| - Gain window –with internal FE | 32±2dB | 2nd DCA full atten min. 3dB 2nd DCA full atten max.30dB |
| - Gain window –without internal FE | 16±2dB |  |
| - Gain controls RF to IF minimum: | 30 dB @ step 1dB |  |
| - Max. Noise Figure [dB] (at 25C): with internal FE | 6dB up to 2GHz 8dB up to 6GHz 9dB up to 11GHz 10dB up to 15GHz 11dB up to 17GHz 12dB up to 18GHz | @ Gain = 20dB |
| Max. Noise Figure [dB] (at 25C) : without internal FE | 11dB up to 2GHz 11dB up to 6GHz 12dB up to 11GHz 13dB up to 15GHz 14dB up to 17GHz 14dB up to 18GHz | @ Gain = 15dB 3dB degradation |
| - Max input power without damage for internal FE | 36 dBm @CW (25°C) | (Limiter TGL2217-SM Spec) |
| - Max input power –without internal FE | Not required Limiter | The limiter will remove as well the first LNA |
| - P1dB Output compression Min. | 7dBm Design goal 10dBm After ELTA testing A/D is 10dBm | A/D FS is 6[dBm] @ Gain = 30dB |
| P1dB Input for first amp | +5dBm for 0.5-6GHz -6dBm for 6-18GHz | With internal FE |
| - Max IF output power [dBm] | +13dBm After testing Elta is +14dBm | A/D Undamaged |
| - IP3 Min. Output (dBm) | +16 dBm @ Conversion 22dBm @ direct |  |
| - Spurious - IN Band in to IN band out | For Direct: 70dBc For Conversation: 60dBc | 1 RF input where abs(Fin-Ft) <= 1.25 GHz, Pin <= -40dBm, abs(Fspur-3.75) GHz < 1.25 GHz IN-IN spurs (typically higher order intermods) |
| - IF Second harmonic in band (IN-IN) | 35 dBc @ Conversion 45 dBc @ IF Range #1 @ #2 (No conversion) | Up to 1dBc For any input frequency 0.5-18GHz |
| - selectivity and Image rejection: OUT of Band ±300MHz to IN band out | 65 dBc min. at 300 MHz for any injected signal at 300 MHz away from the selected input operate frequency range. | 1 RF input where abs(Fin-Ft) > 1.25 GHz, Pin <= -40dBm, abs(Fspur-3.75) GHz < 1.25 GHz determined by the input filter. before the first mixer. OUT-IN spurs (image is typically worst case) |
| - Spurious - OUT of Band ±300MHz to OUT band | 65 dBc | 1 RF input where abs(Fin-Ft) > 1.25 GHz, Pin <= -40dBm, abs(Fspur-3.75 GHz )> 1.25 GHz OUT-OUT spurs (typically harmonics of output) |
| - Spurious - IN Band in to OUT band out | 65dBc | 2 or more RF inputs where abs(Fin-Ft) < 1.25 Ghz for all tones - or all but one tone. Pin <= -40dBm total for all tones abs(Fspur-3.75) GHz > 1.25 GHz IN-OUT spurs (typically driven by IF filter) |
| - Independent Spurious at Output -without receiving signals | -75 dBm (@ Conversion, Design Goal -80dBm) -80dBm @ direct | For any input frequency setting max gain Any products, Such as LO products, Clock. |
| - Spurious (Amplitude Mode) | 5 dB Degradation related to the phase mode |  |
| - Total noise level output | -114dBm+N/F+ Gain+ BW 500M + 2dB | At any gain setting |
| - Inputs/output VSWR (max) | 2.0:1 (90%)2.5:1 |  |

| - 8 RF path Electrical unit Requirements phase match or stand alone at separated frq. |  |  |
| --- | --- | --- |
| - Common LO (synthesizer) or independent for each path (8 path) | Each 8CH | One LO or 8 LO will be defined by command. Will be design for 8. Will be decided to assembled upon project |
| - Phase matching over any 50MHz | ±17° |  |
| Phase matching over any 50MHz With look up calibration | ±4° |  |
| - Phase matching over 500MHz | ±30° |  |
| Phase matching over 500MHz With look up calibration | ±8° | Over temperature |
| Phase matching stability | ±4° | Over time |
| - Amplitude matching | ±3dB |  |
| Amplitude matching with look up calibration | ±1dB |  |
| - Amplitude matching stability | ±0.25dB | day to day repeatability |
| - Isolation between channels at the same frequency setting | 40dB |  |
| - Isolation between channels at different frequency setting (Amp Mode) | 50dB |  |
| - Voltages | TBD will be defined by vendor | Comply with MIL-STD-704D power bus specification |
| - Peak Power consumption (power supply capability) | TBD | For the PS. Design |
| - Inrush current | 150% | From peak |
| - Power consumption | Up to 70W ( 8ch.) Up to 45W (4 ch.) |  |
| - Control | SPI or NADCOM LVTTL | IDD DOC Independent to each channel Should be design to support both of them |
| - BIT Status | Lock Detects Temperature Version | Voltage Rails option |
| - Local Oscillators / Reference |  |  |
| - LO sources | 8 | to meet amp mode |
| - Lock to Ext 50 ohm sine Reference |  |  |
| - Frequency Tuning Step | <= 0.1 GHz |  |
| - Frequency Tuning Speed | <= 0.2 mS |  |
| - Reference Frequency | 10 @ 50 ohm sinus | Internal reference in case not external Ref – automatically |
| - Reference Input power | 7±1 dBm |  |
| - Operation on Internal Reference |  |  |
| - Internal Reference Accuracy (at room) | ±0.1 ppm |  |
| - Internal Reference Stability (over temp) | ±0.2 ppm | For 5minutes(±0.1ppm) |
| - Phase Noise internal REF | Similar to PDC/ LCG |  |
| - Phase Noise external (@ ext ref | 20LOG (LO/Ref) + 4dB dBc/Hz | TBD (need ref spec) |
| - Integrated phase noise (100Hz->10MHz) | TBD (need ref spec) |  |
| - CAL RF Source output port |  |  |
| - Frequency Range [GHz] | 0.5-18GHz |  |
| - Frequency tuning step | Minimum 25MHz | Target 4MHz |
| - output power (typ, on) | -30±3 dBm | Internal Data Storage (for calibration info) |
| - power control | 50dB @ step 1dB | Will be discussed |
| - Frequency accuracy | Same as reference | With the internal oscillator |
| - Bit/Cal modulation | Internal 25µ or External ON/OFF 40dB | Internal mod can be change |
| - Internal Data Storage (for calibration info) | ±3 dB |  |
| - Calibration source output power (off) | <=-90 dBm | Consider: set to extreme FRQ. Max att. Voltage off |
| - Spurious | 40dBc |  |

| - Clock to A/D |  |  |
| --- | --- | --- |
| - Frequency | 250 MHz sinus |  |
| - POWER | 6- 8 dBm Sinus |  |
| - BIT (Built in Test) |  |  |
| - FAIL indication | SEE IDD |  |
| - LO lock detect read | Lock |  |
| - Temperature read | -50 to 90º |  |
| - Over Temp Shut Down | Temp > TBD | 5º before unit damage |
| - Firmware Version read | XXX |  |
| - Voltage Levels read | OK/FAIL |  |
| - S/N | XXX |  |

  - Mechanical requirements

According to SOCD

| No | Main Function | Name | SOCD |
| --- | --- | --- | --- |
| 1 | 2GHz WB Miniature Octagon Frequency Converter | MOFC or M8FC | 3019Y206-001 |

    - Outline Drawing

All interface dimensions shall be in accordance with SOCD

    - RF Connectors:

According to SOCD

    - DC and control connector:

According to SOCD number

All connections that are not sealed shall be capped with push-on caps to prevent damage and the entrance of moisture or foreign materials during shipment and storage.

  - Weight

According to SOCD

Weight status shall be closely monitored during the development phase of this project.

      - Cooling

TBD

      - Sealing

N/A  - The Unit is mounted in a sealed box

  - Design and contraction

    - Material and Parts

The material and parts shall be as specified in 2.1. When a definite material or part is not specified, the manufacturer shall use one which will enable the component to meet the performance & environmental requirements of this specification. A documented derating policy shall be uniformly applied in the selection and application in accordance with MIL-HDBK-454 guideline 18 or similar.

Components will be selected from manufacturers having quality controlled production line.

Manufacturers presently listed in the QML/QPL and/or certified/approved by known BCP (Best Commercial Practice) used and/or certified/approved in accordance with relevant Non-Governmental Standards, will be preferred.

    - SOLDER LEAD FREE

The product shall not use RoHs components and no lead free soldering (design goal)

    - Aluminum Alloys

Aluminum alloy plates and sheets shall conform to QQ-A-200 or QQ-A-250 (AL 6061 T6 or T651 is preferred) Aluminum alloy casting shall conform to QQ-A-596 QQ-A-601 or MIL-C-11866.

    - Brass

Brass shall conform to alloy 360 of QQ-B-626

    - Dissimilar Metals

Unless suitably protected against electrolytic corrosion dissimilar metals as defined in MIL-STD-889 shall not be in intimate contact.

    - Fungus Inert Material

Materials used in the construction of these units shall be fungus inert IAW 
MIL-HDBK-454 guideline 4.

    - Arc-Resistant Materials

Arc-resistant materials shall be IAW MIL-HDBK- 454 guideline 26.

    - Flammable Materials

Flammable materials shall be in accordance with MIL-HDBK-454 guideline 3.

    - Adhesives

Adhesives shall be in accordance with MIL-HDBK-454 guideline 23. Their selection shall be based on their strength in the appropriate failure mode (i.e. shear, peal etc.) at the extreme temperature and rate of change of temperature.

    - Stripline Transmission Lines

Stripline transmission material shall be in accordance with MIL-P-13949 Type Non-woven (Duroid 5880 or 5870 are preferred) bonded stripline circuit boards are preferred to boards assembled by eyelets or screws. Connections between stripline assemblies and coaxial lines shall be designed to accommodate the maximum predicted thermal expansion and contraction of the coax center conductor.

    - ESD (Electro-Static Discharge)

ESD control shall be in accordance with MIL-HDBK-454 guideline 75.

    - Threaded Parts:

All threaded parts shall have screw threads in the unified screw thread series in accordance with handbook FED-STD-H28.

    - Soldering

Soldering shall be in accordance with MIL-HDBK-454 guideline 5.

    - Finish

According to SOCD.

    - Marking

The unit shall be marked in accordance with MIL-STD-1285 Method I.

The manufacturer shall use a method which will provide legible and permanent marking for the life of the item.

Marking areas location shall be accordance with SOCD

    - Name plate marking

Name plate marking shall be in accordance with SOCD

    - Resistance to solvents:

Units shall be resistant to solvents in accordance with MIL-STD-202 method 215.

    - Resistance to soldering heat

Unit shall be resistant to soldering heat in accordance with MIL-STD-202 method 210.

    - Operating Positioning:

The unit shall operate reliably in any position.

  - Environmental Requirements

The unit shall perform as specified when operating in the operating environmental conditions as defined below. In the non-operating condition, the equipment is powered down. The equipment shall withstand long periods of exposure to the temperature extremes and other operating and non-operating environmental conditions specified herein. The following requirements represent the worst case among the ground environmental conditions.

It is the Manufacturer responsibility to provide the relevant corrective actions in case the unit will fail during environmental qualification tests and actually apply these corrective actions on MOFC.         Qualification test shall be conducted by Manufacturer (vendor).

    - General Environmental specification

| Requirements | Environment |
| --- | --- |
| MIL-STD-810F, Method 520.2, Procedure III Ambient Temperature: -46 º÷ +55ºC. Figure 10 describes two main sides of unit. Each side is exposed to different temperature conditions (as noted on this figure). | Operating Temperature |
| MIL-STD-810F, Method 520.2, Procedure III -46°C to +71°C | Non-Operating Temperature |
| MIL-STD-810F, Method 520.2, Procedure III: 45Kft | Operating Altitude |
| MIL-STD-810F, Method 520.2, Procedure III: 45Kft | Non-Operating Altitude |
| MIL-STD-810F, Method 507.4 Up to 100% RH at temperature up to +60°C | Humidity |
|  | Vibration Functional & Endurance |
| MIL-STD-810F, Method 514.5, Figure: 514.5C-9 modified as given in Figure 11 and Table 6, given below | - UAV |
| RTCA/DO160D, Section 8, Figure 8-1, Curve B (1.48 Grms) and Figure 8-4, Curve B1 (2.09 Grms) | - Jet aircraft (cabin) |
| MIL-STD-810G, Method 528 (Functional & Endurance) | - Ship |
| MIL-STD-810F, Method 516.5, Procedure I, 20g for 11 msec, Saw Tooth | Mechanical shock Functional |
| MIL-STD-810F, Method 509.4, 5% NaCl | Salt fog |
| MIL-STD-810F, Method 508.5 | Fungus |
| MIL-STD-810F, Method 510.4, Procedure I | Dust |
| MIL-STD-810F, Method 513.5, Procedure II, 10g along each axis direction | Acceleration Functional |
| MIL-STD-810F, Method 506.4, Procedure III | Drip |
| MIL-STD-810G, Method 511.4 | Explosive Atmosphere |

"Side B" – Forced temperature: TBD

"Side A" – Ambient temperature

Figure 10: "Side A" and "Side B" of MOFC – Environment Temperature

Figure 11: UAV Vibration Envelope

| Endurance Specification Requirements, g2/Hz | Functional Specification Requirements, g2/Hz | Frequency, Hz | Parameter |
| --- | --- | --- | --- |
| 0.135 | 0.1 | 83-114 | F0 |
| 0.034 | 0.025 | 166-228 | F1 |
| 0.016 | 0.012 | 249-342 | F2 |
| 0.013 | 0.01 | 15-2000 | W0 |

Table 6: UAV Vibration Envelope Parameters

    - Environmental testing and analysis

The MOFC shall be subjected to the following process:

      - Environmental Tests

- ESS for every manufactured unit.

- Development Tests Document

- POD – Proof of Design

- QTP – Qualification Tests.

      - QA Analysis

- Reliability Analysis

- Stress Analysis

- QAP – Quality Assurance Plan Document

    - Environmental Requirements

      - Temperature and Altitude

The CDRP shall withstand altitude from sea level to 45,000 ft and temperature range as following:

Operating: -46ºC to +55ºC

Non-Operating: -46ºC to +71ºC

Figure 9 describes two main sides of MOFC. Each side is exposed to different temperature conditions (as noted on this figure).
The unit will be tested in accordance with MIL-STD-810F, Method 520.2, modified procedure III.

      - Humidity

The unit shall withstand, in an operating and non-operating condition (including condensation condition), exposure to a relative humidity up to 100% and temp 30°C to 60°C, per MIL-STD-810F, Method: 507.4, for 10 Cycles.

      - Salt fog

The unit shall withstand, in both: operating and non-operating condition, exposure to salt sea atmosphere, as per MIL-STD-810F, Method: 509.4, Salt solution of 5±1% Concentration 24 hrs exposure & 24 hrs drying constitutes one cycle, with a total of 2 cycles (96 hrs).

      - Fungus

All selected parts and materials used in the unit shall be non-nutrient to fungus growth as encountered in tropical climates. Per MIL-STD-810F, Method: 508.5.

      - Dust

The unit shall be functionally tested to meet the electrical requirements, before and after exposure to dust in atmosphere.

The level of exposure to dust shall be in accordance with MIL-STD-810F, Method 510.4, procedure I (blowing dust).

      - Vibration

The unit shall operate within the specified performance requirements when subjected to functional levels of random vibration, as described below.

The unit shall withstand the functional vibration level (as described above) without any damage.

In addition, the unit shall withstand Endurance random vibration level, without any damage and shall be able to perform as specified after completing Endurance vibration.

Functional vibration and Endurance vibration, shall be conducted in accordance with following three different platforms requirements:

| - UAV |  | MIL-STD-810F, Method 514.5, Figure: 514.5C-9 modified as given in Figure 10 and Table 1 (Functional & Endurance). |
| --- | --- | --- |
| - Jet aircraft (cabin) |  | RTCA/DO160D, Section 8, Figure 8-1, Curve B (1.48 Grms) and Figure 8-4, Curve B1 (2.09 Grms). |
| - Ship (Mast) | MIL-STD-810G, Method 528 (Functional & Endurance). |  |  |

      - Functional Acceleration

The unit, normally mounted in operating mode, shall not suffer structural/mechanical damage being subjected to an acceleration stress level of 10g along each of the 3 mutually perpendicular axes (total of 6 Acceleration tests) for 1 minute along each direction.

Functional Acceleration requirements per MIL-STD-810F, Method: 513.5, Procedure II.

      - Functional Shock

The unit, normally mounted in operational mode, shall be withstand 18 sawtooth mechanical shocks (3 shocks in each of six directions) with a peak acceleration of 20g for a duration of 11±1 msec each. Functional Shock requirements per MIL-STD-810F, Method: 516.5, Procedure I.

      - Explosive Atmosphere

The unit shall be design to comply with the requirements for explosive atmosphere, in accordance with the guidelines of MIL-STD-810G, Method 511.4.

      - Drip proof

The unit shall withstand drip caused by rain penetration. The level of exposure to drip shall be in accordance with MIL-STD-810F, Method 506.4, Procedure III.

  - EMI and EMC

    - Bonding

Bonding refers to the process by which a very low path of electric current is established between two metallic objects

    - Bonding Unit's body:

The UNIT body will be bonded to the structure by its body and the mounting screws (e.g. the mating interface between the UNIT body and structure will be free from nonconductive material). 
For Off-the-Shelf RF UNIT when it is not possible to use its body for bonding, the bonding can be implemented by its external coax cables.,

The bonding design will assure a dc bond resistance of 0.0025 Ohm to the structure.

    - Bond for shielding:

The resistance across joint or seams in metallic members required to provide EMC shielding (e.g. cover) will have a dc resistance of 0.0025 Ohm.

    - Bond for connectors

For the interface external connectors, the resistance between the connector body and the SRI structure will have a dc resistance of 0.0025 Ohm.

    - Bond for EMI filters

The resistance between the EMI filters body and the UNIT structure will have a dc resistance of 0.0025 Ohm.

    - Electromagnetic Compatibility

The unit shall be designed to comply with MIL-STD-461F as describe below

| Ref. | Requirement | Test Description | Limits and remarks |
| --- | --- | --- | --- |
| 1. | CE102 | Power leads 10 KHz to 10 MHz | Limit CE102-1 Basic curve |
| 2. | CS101 | Conducted Susceptibility, Power leads, 30 Hz to 150 kHz | Limit CS101-1 |
| 3. | CS114 | Conducted Susceptibility, Bulk Cable Injection, 10 KHz to 200 MHz | Limit CS114-1, 10 kHz to 2 MHz - Curve #3, 2 MHz to 200 MHz - Curve #5 |
| 4. | CS115 | Conducted Susceptibility, Bulk Cable Injection, Impulse Excitation | Limit CS115-1 (at 30Hz rate for 1 minute) |
| 5. | CS116 | Conducted Susceptibility, Damped Sinusoidal Transients, Cables, 10KHz to 100MHz. | Limit CS116-1 and CS116-2; Imax = 10 amperes |
| 6. | RE101 | Radiated Emissions, Magnetic Field, 30 Hz to 100 kHz | As Figure RE101-2 |
| 7. | RE102 | Radiated Emission, Electric Field, 10KHz to 18GHz. | Limit RE102-4, Navy Fixed & Air Force limit |
| 8. | RS101 | Radiated Susceptibility, Magnetic Field, 30 Hz to 100 kHz | As Figure RS101-1 |
| 9. | RS103 | Radiated Susceptibility, Electric Fields, 2MHz to 18GHz | As Table VII. 200 V/m @ Out of each band ±5%, 0.5-2, 2-18 and 6-18 GHz, 50 V/m @ In band. |

  - Reliability and Quality Requirements

    - Predicted MTBF

The CDRP MTBF shall be not less than 20000 hours.

Environmental Conditions for MTBF prediction:

AUC (Airborne Unabated Cargo) conditions as defined in MIL-HDBK-217F. The ambient temperature shall be 49°C for this calculation.

- The MTBF shall be established by appropriate data/sources, and based on MIL-HDBK-217F.

    - Storage Life

- At any time during and after storage and at any combination of environmental conditions, as specified in this specification, the unit shall meet all the specified requirements of this document without the need for replacement, adjustment or maintenance of a component, part or subassembly.

- The product will be stored in regular storage rooms of ambient temperature of -20C to +70C and relative humidity up to 90%.

    - Operating Service Life

- The unit shall have a useful life of not less than fifteen (15) years for any applicable combination of operation, endurance, performance and storage.

    - Maintainability

- The Mean Time to Repair (MTTR) of the LRU includes the time for fault isolation, removal/replacement, and verification and meets the following requirements:

If a fault lies within the maintenance level, the LRU design will enable to correct it by LRU replacement within MTTR of 30 (thirty) minutes.

- Maximum Time to Repair the unit, with confidence level of 95% shall be less than 1 hours.

- The unit design and construction shall incorporate features such that it shall be mechanically and electrically impossible to install it incorrectly and to attach cables, electrical plugs and other such items in an improper manner.

- QUALITY ASSURANCE PROVISIONS

  - Responsibility for inspection

Unless otherwise specified in the contract or purchase order, the supplier is responsible for the performance of all inspection requirements as specified herein. The supplier may use his own or any other facilities suitable for the performance of the inspection requirements specified herein, unless disapproved by Buyer.

    - Source Inspection

Buyer reserves the right to perform source inspection at the manufacturer's facilities. When source inspection is called for in the purchase order, the manufacturer shall advise the Buyer at least 15 days prior to the start of tests when they are scheduled to be performed. Unless otherwise specified in the SOW.

    - Non-Performance of Test/Inspections

Non-performance of tests and inspections to establish compliance with the requirements specified herein shall not exempt the manufacturer from providing units that do-indeed comply with all the specified requirements.

    - Test Equipment and Inspection Facilities

Test equipment and inspection facilities shall be of sufficiently accurate quality and quantity, to assure that performance of required inspection can be established and maintained by the manufacturer. A calibration system shall be maintained to control the accuracy of test equipment employed and shall be in  accordance with MIL-C-45662.

  - Acceptances Test

All production units shall be subjected to Acceptance Test.

The first unit will be subjected to First Article Inspection process. 
The supplier shall inform the customer at least two weeks before start of acceptance test. The customer reserves the right to survey such acceptance tests.

Acceptance Test consists of:

- ESS

- Mechanical and Visual Inspection

- Electrical Test (according to approved ATP) 
Test over entire operating temperature range (Exact temperature to be specified by vendor in ATP)

    - ESS

Each unit delivered in production shall pass Environmental Stress Screening as defined below.

ESS consists of:

- Thermal cycles

- Random vibration

    - Thermal Cycles

The unit shall pass 8 thermal cycles between -40°C to +85°C. Rate of temperature change shall be at least 5°C/min and dwell at each extreme temperature at least 1 hour.

The unit must be continuously operated in the transition period from cold to hot and during hot period dwell. At the end of each hot period a functional test must be performed. (The actual test will be mutually agreed based on para 8 - Verification Matrix)

In the first cycle a functional test must be done in cold dwell period.

The last two cycles must be failure free. If failures occur during failure free period, the test time must be prolonged until the failure free period is achieved.

    - Random Vibration

The unit shall be exposed to 10 minutes random vibration in the axis normal to the plane of the module.

Test level 6 GRMS in accordance with the following figure:

G2/Hz

0.04

3dB/OCT

-3dB/OCT

0.01

350

2000

80

20

Frequency [Hz]

Figure 12: Random Vibration

During vibration the unit operation will be monitored.

Based on these considerations the manufacturer will submit a detailed ESS test procedure for customer approval.

    - Mechanical and Visual Inspection

- Dimensions per SOCD

- Weight per SOCD

- Finish

- Workmanship

- f) Marking

      - Electrical Tests

The manufacturer will perform testing on each module delivered against the contract in accordance with an ATP. The ATP shall test, at least, the tests according to ATP column in paragraph 8.

      - Rejection and Re-Test

The unit that has failed to meet the acceptance tests of this specification shall be rejected. Non-compliance with any of the specification shall be corrected (rework or parts replacement) such that full compliance is achieved. Full particulars concerning the rejection and corrective actions shall be submitted with the unit to Buyer.

Whenever a submitted lot is rejected, no item(s) on hand or later produced shall be submitted for acceptance until the extend and cause of failure are determined. The rejected item(s) may be reworked or have parts replaced to correct the defect and resubmitted for acceptance.

## EMC and EMI Assurance and Tests

    - Bonding

Bonding will be assured by inspection with documentation covering the requirements as defined above.

    - Electromagnetic Compatibility

Tests as are required as defined above.

- RESPONSIBILITY OF SELLER

  - Implementation of changes

After the shipment of the first unit under this specification, the manufacturer shall not implement any change which affects the input/output characteristics, performance, shape, size, weight, reliability, substitutability, interchangeability, marking, finish external appearance of the manufacturing process of the unit without written advance notice to and approval by Buyer.

- PREPARATION FOR DELIVERY

Unit shall be packaged and packed in accordance with standard practices to provide protection from hazards of contamination and damage encountered in general handling and overseas shipment.

- APPROVED SOURCES

- VERIFICATION MATRIX

Table 7: Verification Matrix

| Parameter |  | ATP | temp | ESS | QTP | Eng. Test (3) | Analysis Test (9) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Electrical channel Requirements |  |  |  |  |  |  |  |
| Input Frequency Range [GHz] |  | x |  | x | x |  |  |
| Output IF Frequency Range GHz] |  | x |  | x | x |  |  |
| LO/CAL Frequency Range [GHz] |  |  |  |  |  | x | x |
| LO/stability |  |  |  |  |  | x | x |
| Frequency tuning step |  | x |  |  |  |  |  |
| Phase Noise |  |  |  |  |  | x | x |
| Gain controls RF to IF minimum: |  | x | x |  |  |  |  |
| Max. Noise Figure [dB] (at 25C) |  | x | x |  |  |  |  |
| Max input power without damage |  |  |  |  |  |  | x |
| 1dB compression Min. A/D (Vpp @ 100Ω) RF Output [dBm] |  | x |  |  |  |  |  |
| Max output power (Vpp @ 100Ω) RF Output [dBm] |  | x |  |  |  |  |  |
| IP3 Min. Output (dBm) |  |  |  |  |  | x |  |
| Intermodulation Output Level at -6dBm per tone at output, In Band |  |  |  |  |  | x |  |
| Intermodulation Output Level, at -10 dBm per tone at input (Out of Band FC±600MHz) |  | x |  |  |  |  |  |
| Spurious Output Level |  | x |  |  |  |  |  |
| Second harmonic of output signal in band |  | x |  |  |  | x |  |
| Image rejection: |  |  |  |  |  |  |  |
| Undesired signals rejection - |  | x |  |  |  |  |  |
| Total noise level |  |  |  |  |  | x |  |
| Inputs/output VSWR (max) |  | x |  |  |  |  |  |
| External Input Reference: |  |  |  |  |  |  |  |
| BIT/ CAL CW & modulation |  | x |  |  |  |  |  |
| BIT ON/OFF ratio |  | x |  |  |  |  |  |
| Switching Times |  |  |  |  |  | x | x |
| Common LO (synthesizer) |  |  |  |  |  |  |  |
| Internal calibration |  |  |  |  |  |  |  |
| Phase matching over any 50MHz (after calibration), including BIT path errors |  | x |  |  |  |  |  |
| Phase matching over 500MHz (after calibration), including BIT path errors |  | x | x |  |  |  |  |
| Amplitude matching (after calibration) |  | x |  |  |  |  |  |
| Switch select between sides |  |  |  |  |  |  |  |
| Isolation between directions (left -> right) |  | x |  |  |  |  |  |
| Isolation at the same directions (channel 1-> channel 2,3,4) |  | x |  |  |  |  |  |
| FAIL indication |  | x |  |  |  |  |  |
| Voltage |  |  |  |  |  | x |  |
| Peak Power consumption (power supply capability) |  |  |  |  |  | x |  |
| Inrush current |  |  |  |  |  | x |  |
| Power dissipation (typical, including power supply efficiency, excluding A2D) |  |  |  |  |  | x |  |
| Power dissipation (typical, including power supply efficiency, including A2D) |  |  |  |  |  | x |  |
| Power dissipation (typical, including power supply efficiency, excluding A2D) |  |  |  |  |  | x |  |
| Power dissipation (typical, including power supply efficiency, including A2D) |  |  |  |  |  | x |  |
| Control |  |  |  |  |  |  |  |
| BIT capability - RF Status |  | x |  |  |  |  |  |
| CAL internal RF signal injection |  |  |  |  |  |  |  |
|  | LO/CAL |  |  |  |  |  |  |
| CAL Frequency Range [GHz] |  | x | x |  |  |  |  |
| Frequency tuning step |  | x |  |  |  |  |  |
| Frequency accuracy |  |  |  |  |  | x |  |
| Bit/Cal modulation |  | x |  |  |  |  |  |
| REF input |  |  |  |  |  |  | x |
| Spurious |  | x |  |  |  |  |  |
| Phase Noise |  |  |  |  |  | x |  |
| Phase Matching and tracking |  | x |  |  |  |  |  |
| LO |  |  |  |  |  |  |  |
| Frequency accuracy |  |  |  |  |  | x |  |
| Spurious |  |  |  |  |  | x |  |
| Weight |  |  |  |  |  | x |  |
| Finish |  | x |  |  |  |  |  |
| Marking |  | x |  |  |  |  |  |
| Environmental |  |  |  |  |  | x |  |
| EMI and EMC |  |  |  |  |  | x |  |
| Workmanship |  | x |  |  |  |  |  |